VLSI Internship – Task 3 Report

🟣 Title

Implementation of Sequential Circuits using Verilog HDL

🟣 Objective

To design and simulate basic sequential circuits such as D Flip-Flop, JK Flip-Flop, 4-bit Register, and 4-bit Counter using Verilog HDL and verify their functionality using waveform analysis.

🟣 Theory

Sequential circuits are digital circuits whose output depends on both present inputs and previous states. These circuits use memory elements like flip-flops and operate based on clock signals.

Key Concepts:

- Clock Signal: Controls timing of operations

- Flip-Flop: Basic memory element

- Register: Stores multiple bits

- Counter: Counts clock pulses

Unlike combinational circuits, sequential circuits have memory and depend on past values.

🟣 1. D Flip-Flop

🔹 Design Code:

module d_flipflop(
    input clk,
    input D,
    output reg Q
);

always @(posedge clk)
begin
    Q <= D;
end

endmodule

🔹 Testbench:

module tb;

reg clk;
reg D;
wire Q;

d_flipflop uut(.clk(clk), .D(D), .Q(Q));

always #5 clk = ~clk;

initial begin
    $dumpfile("dump.vcd");
    $dumpvars(0,tb);

    clk = 0;
    D = 0;

    #10 D = 1;
    #10 D = 0;
    #10 D = 1;

    #20 $finish;
end

endmodule

🔹 Output:

Q follows D at the positive edge of clock.

🟣 2. JK Flip-Flop

🔹 Design Code:

module jk_flipflop(
    input clk,
    input J,
    input K,
    output reg Q
);

always @(posedge clk)
begin
    case({J,K})
        2'b00: Q <= Q;
        2'b01: Q <= 0;
        2'b10: Q <= 1;
        2'b11: Q <= ~Q;
    endcase
end

endmodule

🔹 Testbench:

module tb;

reg clk;
reg J, K;
wire Q;

jk_flipflop uut(.clk(clk), .J(J), .K(K), .Q(Q));

always #5 clk = ~clk;

initial begin
    $dumpfile("dump.vcd");
    $dumpvars(0,tb);

    clk = 0;
    J = 0; K = 0;

    #10 J=0; K=0;
    #10 J=0; K=1;
    #10 J=1; K=0;
    #10 J=1; K=1;
    #10 J=1; K=1;

    #20 $finish;
end

endmodule

🔹 Output:

- 00 → No change

- 01 → Reset

- 10 → Set

- 11 → Toggle

🟣 3. 4-bit Register

🔹 Design Code:

module register4(
    input clk,
    input [3:0] D,
    output reg [3:0] Q
);

always @(posedge clk)
begin
    Q <= D;
end

endmodule

🔹 Testbench:

module tb;

reg clk;
reg [3:0] D;
wire [3:0] Q;

register4 uut(.clk(clk), .D(D), .Q(Q));

always #5 clk = ~clk;

initial begin
    $dumpfile("dump.vcd");
    $dumpvars(0,tb);

    clk = 0;
    D = 4'b0000;

    #10 D = 4'b1010;
    #10 D = 4'b0101;
    #10 D = 4'b1111;
    #10 D = 4'b0001;

    #20 $finish;
end

endmodule

🔹 Output:

Q stores the 4-bit input D at every clock edge.

🟣 4. 4-bit Counter

🔹 Design Code:

module counter4(
    input clk,
    output reg [3:0] count
);

always @(posedge clk)
begin
    count <= count + 1;
end

endmodule

🔹 Testbench:

module tb;

reg clk;
wire [3:0] count;

counter4 uut(.clk(clk), .count(count));

always #5 clk = ~clk;

initial begin
    $dumpfile("dump.vcd");
    $dumpvars(0,tb);

    clk = 0;

    #100 $finish;
end

endmodule

🔹 Output:

Counter increments on every clock pulse.

🟣 Result

All sequential circuits were successfully designed and simulated using Verilog HDL. The outputs were verified using waveform analysis.

🟣 Conclusion

This task helped in understanding the working of sequential circuits, including flip-flops, registers, and counters. It also improved practical knowledge of Verilog HDL and simulation tools.

🟣 Tools Used

- EDA Playground

- Icarus Verilog

- EPWave